Open Ended Experiment Report

On

Effects of Pre-emphasis of an Input Signal

Communication Engineering Laboratory (EC29002)

Submitted by

Anubrata Mukhopadhyay (2230311)

B.Tech in Electronics and Computer Science Engineering

School of Electronics Engineering

Kalinga Institute of Industrial Technology Deemed to be University

Bhubaneswar, India

November 2024

Aim of the Experiment : To observe the effects of pre-emphasis on a given input signal.

Components Required :

| NAME | SPECIFICATION | QUANTITY |
| --- | --- | --- |
| Transistor (BC 107) | ft = 300MHz Pd = 1W Ic(max) = 100mA | 1 |
| Resistors | 10 KΩ, 7.5 KΩ, 6.8 KΩ | 1 each |
| Capacitors | 10 nF 0.1 µF | 1 2 |
| CRO | 20 MHz | 1 |
| Function Generator | 1 MHz | 1 |
| Regulated Power Supply | 0-30 V, 1 A | 1 |

THEORY : The presence of noise is also an issue in FM and we know that noise usually has higher amplitude and higher frequency. When the amplitude of a high-frequency noise is higher than the current component in the modulation signal, it causes high-frequency interference. To deal with this issue, most FM circuits use a technique called pre-emphasis during transmission and de-emphasis during receiving. Pre-emphasis and de-emphasis circuits are commonly used in FM transmitters and receivers to improve the output signal-to-noise ratio. Pre-emphasis and De-emphasis are used to improve the fidelity of FM transmission of the audio signal.

CIRCUIT DIAGRAM :

PROCEDURE : 1. Connect the circuit as per circuit diagram as shown in Fig.1.

2.Apply the sinusoidal signal of amplitude 20mV as input signal to pre-emphasis circuit.

3. Then by increasing the input signal frequency from 500 Hz to 20KHz, observe the output voltage (vo​) and calculate gain (20 log (vo​/vi​)).

4. Plot the graph between gain Vs frequency.

SAMPLE READINGS :

Table 1 : Pre-emphasis  (Vi = 20mV)

| Frequency | Vo (in mV) | Gain in dB |
| --- | --- | --- |
| 500 Hz | 20 | 0 |
| 1 KHz | 30 | 3.52 |
| 2 KHz | 40 | 6.02 |
| 3 KHz | 50 | 7.96 |
| 4 KHz | 65 | 10.23 |
| 6 KHz | 80 | 12.04 |
| 8 KHz | 95 | 13.52 |
| 12 KHz | 115 | 15.21 |
| 16 KHz | 130 | 16.26 |
| 20 KHz | 145 | 17.22 |

PLOT :

BREADBOARD CONNECTIONS :

INPUT SIGNAL :

PRECAUTIONS :

- Check the connections before giving the power supply.

- Observations should be made carefully.

CONCLUSION :

Pre-emphasis was performed successfully on an input signal of 20 mV and ten different readings were taken for different frequencies of the input signal. The graph for pre-emphasis was created based on these values, which showed an increase in gain with increase in frequency of input signal.